FAST NUCES — ISLAMABAD

Department of Computer Science

Computer Networks

Final Project Report

Submitted by

Alishba Inam

Student ID: 24i-2036

Lab Teacher: Mam Rubab Zainab

Date of Submission:  6th May 2026

# Table of Contents

Table of Contents	2

1.  Introduction & Objectives	5

Project Objectives	5

Assigned Data	5

2.  VLSM Subnetting Calculations	6

2.1  Base Network	6

2.2  Host Requirements (Sorted Largest to Smallest)	6

2.3  VLSM Allocation Table	6

2.4  WAN Link Subnets (/30 — Point-to-Point)	7

3.  Network Topology Overview	8

3.1  Routing Domains	8

3.2  Redistribution Points	8

3.3  Network-to-Gateway Mapping	8

4.  Device Inventory	9

4.1  Routers (All Cisco 2811)	9

4.2  Switches	9

4.3  End Devices	10

4.4  Servers Summary	10

5.  Cabling & Port Mapping	12

5.1  Cable Types Used	12

5.2  WAN Cabling (Router-to-Router)	12

OSPF Area 1	12

EIGRP 5	12

OSPF Area 2	12

RIP	13

5.3  LAN Cabling (Switch-to-Router, all Straight-Through)	13

6.  Routing Protocol Configurations	14

6.1  OSPF Area 1	14

Router4 — Network A Gateway	14

Router1 — Network B+C Gateway (Hub)	14

Router0 — Transit	14

Router2 — OSPF ↔ EIGRP Border (Redistribution)	15

6.2  EIGRP 5	15

Router3 — EIGRP Entry	15

Router5 — Transit	15

Router6 — Transit (No LAN)	16

Router7 — Network D Gateway	16

Router9 — Network E Gateway	16

Router8 — Network F Gateway + EIGRP→OSPF2 Border	17

6.3  OSPF Area 2	17

Router10 — EIGRP ↔ OSPF2 Border (Redistribution)	17

Router11 — OSPF2 Hub	17

Router12 — Network G Gateway	18

Router13 — Network H+I Gateway	18

6.4  RIP (Version 2)	18

Router18 — OSPF2 ↔ RIP Border (Redistribution)	18

Router14 — Core RIP	19

Router15 — RIP Transit	19

Router16 — Network K Gateway	19

Router17 — Network J Gateway	20

7.  Route Redistribution	21

8.  DHCP Configuration	22

8.1  DHCP Server — Server1 (72.8.0.2)	22

Server1 Static IP Configuration	22

8.2  DHCP Pools	22

8.3  DHCP Relay (ip helper-address)	22

9.  DNS, Mail & Web Server Setup	24

9.1  Server0 Configuration — 72.0.0.2 (Network C)	24

9.2  DNS Service	24

9.3  HTTP/Web Service	24

9.4  Email Service (SMTP/POP3)	24

Registered Users	24

9.5  Email Client Configuration (Example: Laptop2)	24

10.  NAT Configuration	26

10.1  NAT Details	26

10.2  NAT Commands on Router17	26

10.3  Verification	26

11.  Access Control Lists (ACLs)	27

11.1  ACL 1: Block ONE Host in Network A from Web Server	27

11.2  ACL 2: Block ALL of Network D from Web Server	27

12.  Testing & Verification	28

12.1  Routing Table Verification	28

12.2  End-to-End Ping Tests	28

12.3  DHCP Verification	28

12.4  Service Tests	28

13.  Conclusion	30

# 1. Introduction & Objectives

This project implements a complex enterprise network consisting of four routing domains interconnected through route redistribution. The network serves 11 subnets across multiple departments using VLSM (Variable Length Subnet Masking) for efficient IP allocation.

## Project Objectives

- Design a multi-area network using VLSM from base network 72.0.0.0/8

- Implement OSPF Area 1, EIGRP AS 5, OSPF Area 2, and RIPv2

- Configure route redistribution between all routing domains

- Deploy a centralized DHCP server for all networks

- Set up Mail, Web, and DNS services

- Apply NAT (Network Address Translation) on a border router

- Implement ACLs to restrict specific traffic flows

## Assigned Data

| Item | Value |
| --- | --- |
| Public IP (for NAT) | 72.195.221.244 |
| Private IP (for NAT) | 1.139.135.189 |
| Base Network | 72.0.0.0/8 |

# 2.  VLSM Subnetting Calculations

## 2.1 Base Network

72.0.0.0/8 — Total available addresses: 16,777,214

## 2.2 Host Requirements (Sorted Largest to Smallest)

| # | Network | Required Hosts | Host Bits (n) | Block Size (2^n) | Usable Hosts | CIDR |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | C | 107,957 | 17 | 131,072 | 131,070 | /15 |
| 2 | A | 101,175 | 17 | 131,072 | 131,070 | /15 |
| 3 | F | 98,305 | 17 | 131,072 | 131,070 | /15 |
| 4 | E | 87,687 | 17 | 131,072 | 131,070 | /15 |
| 5 | H | 81,173 | 17 | 131,072 | 131,070 | /15 |
| 6 | J | 66,518 | 17 | 131,072 | 131,070 | /15 |
| 7 | B | 64,770 | 16 | 65,536 | 65,534 | /16 |
| 8 | I | 57,250 | 16 | 65,536 | 65,534 | /16 |
| 9 | D | 49,116 | 16 | 65,536 | 65,534 | /16 |
| 10 | G | 46,641 | 16 | 65,536 | 65,534 | /16 |
| 11 | K | 36,065 | 16 | 65,536 | 65,534 | /16 |

## 2.3 VLSM Allocation Table

| # | Network | CIDR | Subnet Mask | Network Address | First Usable | Last Usable | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | /15 | 255.254.0.0 | 72.0.0.0 | 72.0.0.1 | 72.1.255.254 | 72.1.255.255 |
| 2 | A | /15 | 255.254.0.0 | 72.2.0.0 | 72.2.0.1 | 72.3.255.254 | 72.3.255.255 |
| 3 | F | /15 | 255.254.0.0 | 72.4.0.0 | 72.4.0.1 | 72.5.255.254 | 72.5.255.255 |
| 4 | E | /15 | 255.254.0.0 | 72.6.0.0 | 72.6.0.1 | 72.7.255.254 | 72.7.255.255 |
| 5 | H | /15 | 255.254.0.0 | 72.8.0.0 | 72.8.0.1 | 72.9.255.254 | 72.9.255.255 |
| 6 | J | /15 | 255.254.0.0 | 72.10.0.0 | 72.10.0.1 | 72.11.255.254 | 72.11.255.255 |
| 7 | B | /16 | 255.255.0.0 | 72.12.0.0 | 72.12.0.1 | 72.12.255.254 | 72.12.255.255 |
| 8 | I | /16 | 255.255.0.0 | 72.13.0.0 | 72.13.0.1 | 72.13.255.254 | 72.13.255.255 |
| 9 | D | /16 | 255.255.0.0 | 72.14.0.0 | 72.14.0.1 | 72.14.255.254 | 72.14.255.255 |
| 10 | G | /16 | 255.255.0.0 | 72.15.0.0 | 72.15.0.1 | 72.15.255.254 | 72.15.255.255 |
| 11 | K | /16 | 255.255.0.0 | 72.16.0.0 | 72.16.0.1 | 72.16.255.254 | 72.16.255.255 |

## 2.4 WAN Link Subnets (/30 — Point-to-Point)

All WAN links use /30 subnets starting at 72.17.0.0:

| WAN | Link | Network | Side-1 IP | Side-2 IP |
| --- | --- | --- | --- | --- |
| W1 | Router4 ↔ Router1 | 72.17.0.0/30 | .1 (R4) | .2 (R1) |
| W2 | Router1 ↔ Router0 | 72.17.0.4/30 | .5 (R1) | .6 (R0) |
| W3 | Router0 ↔ Router2 | 72.17.0.8/30 | .9 (R0) | .10 (R2) |
| W4 | Router2 ↔ Router3 | 72.17.0.12/30 | .13 (R2) | .14 (R3) |
| W5 | Router3 ↔ Router5 | 72.17.0.16/30 | .17 (R3) | .18 (R5) |
| W6 | Router5 ↔ Router6 | 72.17.0.20/30 | .21 (R5) | .22 (R6) |
| W7 | Router6 ↔ Router7 | 72.17.0.24/30 | .25 (R6) | .26 (R7) |
| W8 | Router6 ↔ Router9 | 72.17.0.28/30 | .29 (R6) | .30 (R9) |
| W9 | Router9 ↔ Router8 | 72.17.0.32/30 | .33 (R9) | .34 (R8) |
| W10 | Router8 ↔ Router10 | 72.17.0.36/30 | .37 (R8) | .38 (R10) |
| W11 | Router10 ↔ Router11 | 72.17.0.40/30 | .41 (R10) | .42 (R11) |
| W12 | Router11 ↔ Router12 | 72.17.0.44/30 | .45 (R11) | .46 (R12) |
| W13 | Router11 ↔ Router13 | 72.17.0.48/30 | .49 (R11) | .50 (R13) |
| W14 | Router11 ↔ Router18 | 72.17.0.52/30 | .53 (R11) | .54 (R18) |
| W15 | Router18 ↔ Router14 | 72.17.0.56/30 | .57 (R18) | .58 (R14) |
| W16 | Router14 ↔ Router15 | 72.17.0.60/30 | .61 (R14) | .62 (R15) |
| W17 | Router15 ↔ Router16 | 72.17.0.64/30 | .65 (R15) | .66 (R16) |
| W18 | Router16 ↔ Router17 | 72.17.0.68/30 | .69 (R16) | .70 (R17) |
| W19 | Router15 ↔ Router17 | 72.17.0.72/30 | .73 (R15) | .74 (R17) |
| W20 | Router7 ↔ Router8 | 72.17.0.76/30 | .77 (R7) | .78 (R8) |
| W21 | Router17 ↔ Router14 | 72.17.0.80/30 | .81 (R17) | .82 (R14) |
| W22 | Router1 ↔ Router2 | 72.17.0.84/30 | .85 (R1) | .86 (R2) |

# 3. Network Topology Overview

## 3.1 Routing Domains

| Domain | Protocol | Routers | Purpose |
| --- | --- | --- | --- |
| OSPF Area 1 | OSPF Process 1, Area 1 | Router4, Router1, Router0, Router2 | Campus core with servers |
| EIGRP 5 | EIGRP AS 5 | Router2, Router3, Router5, Router6, Router7, Router9, Router8, Router10 | Distribution layer |
| OSPF Area 2 | OSPF Process 1, Area 2 | Router10, Router11, Router12, Router13, Router18 | Secondary campus |
| RIP | RIPv2 | Router18, Router14, Router15, Router16, Router17 | Remote sites |

## 3.2 Redistribution Points

| Border Router | Connects | Redistribution |
| --- | --- | --- |
| Router2 | OSPF Area 1 ↔ EIGRP 5 | Mutual redistribution between OSPF and EIGRP |
| Router10 | EIGRP 5 ↔ OSPF Area 2 | Mutual redistribution between EIGRP and OSPF |
| Router18 | OSPF Area 2 ↔ RIP | Mutual redistribution between OSPF and RIP |

## 3.3 Network-to-Gateway Mapping

| Network | Subnet | Gateway Router | Interface | Gateway IP | Routing Domain |
| --- | --- | --- | --- | --- | --- |
| A | 72.2.0.0/15 | Router4 | Fa0/1 | 72.2.0.1 | OSPF Area 1 |
| B | 72.12.0.0/16 | Router1 | Fa0/0 | 72.12.0.1 | OSPF Area 1 |
| C | 72.0.0.0/15 | Router1 | Fa0/1 | 72.0.0.1 | OSPF Area 1 |
| D | 72.14.0.0/16 | Router7 | Fa0/0 | 72.14.0.1 | EIGRP 5 |
| E | 72.6.0.0/15 | Router9 | Fa0/0 | 72.6.0.1 | EIGRP 5 |
| F | 72.4.0.0/15 | Router8 | Fa0/0 | 72.4.0.1 | EIGRP 5 |
| G | 72.15.0.0/16 | Router12 | Fa0/0 | 72.15.0.1 | OSPF Area 2 |
| H | 72.8.0.0/15 | Router13 | Fa0/0 | 72.8.0.1 | OSPF Area 2 |
| I | 72.13.0.0/16 | Router13 | Fa0/1 | 72.13.0.1 | OSPF Area 2 |
| J | 72.10.0.0/15 | Router17 | Fa0/0 | 72.10.0.1 | RIP |
| K | 72.16.0.0/16 | Router16 | Fa0/0 | 72.16.0.1 | RIP |

# 4. Device Inventory

## 4.1 Routers (All Cisco 2811)

| Router | Role | WIC-2T Modules | LAN Networks |
| --- | --- | --- | --- |
| Router4 | Network A gateway | 1 | A |
| Router1 | Network B+C gateway, OSPF hub | 2 (3 serial ports) | B, C |
| Router0 | OSPF transit | 1 | None |
| Router2 | OSPF ↔ EIGRP border | 2 (3 serial ports) | None |
| Router3 | EIGRP entry | 1 | None |
| Router5 | EIGRP transit | 1 | None |
| Router6 | EIGRP transit | 2 (3 serial ports) | None |
| Router7 | Network D gateway | 1 | D |
| Router9 | Network E gateway | 1 | E |
| Router8 | Network F gateway, EIGRP→OSPF2 | 2 (3 serial ports) | F |
| Router10 | EIGRP ↔ OSPF2 border | 1 | None |
| Router11 | OSPF2 hub | 1 | None |
| Router12 | Network G gateway | 0 (Ethernet only) | G |
| Router13 | Network H+I gateway | 1 | H, I |
| Router18 | OSPF2 ↔ RIP border | 1 | None |
| Router14 | RIP core | 2 (3 serial ports) | None |
| Router15 | RIP transit | 2 (3 serial ports) | None |
| Router16 | Network K gateway | 1 | K |
| Router17 | Network J gateway | 2 (3 serial ports) | J |

## 4.2 Switches

| Switch | Model | Network | Connected To Router | Port to Router |
| --- | --- | --- | --- | --- |
| Switch1 | Switch-PT | A | Router4 Fa0/1 | Straight-through |
| Switch0 | 2960-24TT | B | Router1 Fa0/0 | Straight-through |
| Switch2 | 2960-24TT | C | Router1 Fa0/1 | Straight-through |
| Switch5 | 2960-24TT | D | Router7 Fa0/0 | Straight-through |
| Switch3 | 2960-24TT | E | Router9 Fa0/0 | Straight-through |
| Switch4 | 2960-24TT | F | Router8 Fa0/0 | Straight-through |
| Switch7 | 2960-24TT | G | Router12 Fa0/0 | Straight-through |
| Switch8 | 2960-24TT | H | Router13 Fa0/0 | Straight-through |
| Switch6 | 2960-24TT | I | Router13 Fa0/1 | Straight-through |
| Switch9 | 2960-24TT | K | Router16 Fa0/0 | Straight-through |
| Switch10 | 2960-24TT | J | Router17 Fa0/0 | Straight-through |

## 4.3 End Devices

| Device | Type | Network | Switch | Switch Port | IP Method |
| --- | --- | --- | --- | --- | --- |
| Laptop3 | Laptop-PT | A | Switch1 | Fa0/1 | DHCP |
| Laptop2 | Laptop-PT | A | Switch1 | Fa0/2 | DHCP |
| Laptop0 | Laptop-PT | B | Switch0 | Fa0/1 | DHCP |
| Laptop1 | Laptop-PT | B | Switch0 | Fa0/2 | DHCP |
| PC0 | PC-PT | C | Switch2 | Fa0/1 | DHCP |
| PC1 | PC-PT | C | Switch2 | Fa0/2 | DHCP |
| Server0 | Server-PT | C | Switch2 | Fa0/3 | Static: 72.0.0.2 |
| PC4 | PC-PT | D | Switch5 | Fa0/1 | DHCP |
| Laptop6 | Laptop-PT | D | Switch5 | Fa0/2 | DHCP |
| PC2 | PC-PT | E | Switch3 | Fa0/1 | DHCP |
| Laptop4 | Laptop-PT | E | Switch3 | Fa0/2 | DHCP |
| PC3 | PC-PT | F | Switch4 | Fa0/1 | DHCP |
| Laptop5 | Laptop-PT | F | Switch4 | Fa0/2 | DHCP |
| PC7 | PC-PT | G | Switch7 | Fa0/1 | DHCP |
| PC8 | PC-PT | G | Switch7 | Fa0/2 | DHCP |
| Server1 | Server-PT | H | Switch8 | Fa0/1 | Static: 72.8.0.2 |
| PC5 | PC-PT | I | Switch6 | Fa0/1 | DHCP |
| PC6 | PC-PT | I | Switch6 | Fa0/2 | DHCP |
| PC10 | PC-PT | K | Switch9 | Fa0/1 | DHCP |
| PC9 | PC-PT | K | Switch9 | Fa0/2 | DHCP |
| Server2 | Server-PT | K | Switch9 | Fa0/3 | Static: 72.16.0.2 |
| PC12 | PC-PT | J | Switch10 | Fa0/1 | DHCP |
| PC11 | PC-PT | J | Switch10 | Fa0/2 | DHCP |

## 4.4 Servers Summary

| Server | Location | IP | Subnet Mask | Gateway | Services |
| --- | --- | --- | --- | --- | --- |
| Server0 | Network C (Switch2) | 72.0.0.2 | 255.254.0.0 | 72.0.0.1 | DNS, HTTP, SMTP/POP3 |
| Server1 | Network H (Switch8) | 72.8.0.2 | 255.254.0.0 | 72.8.0.1 | DHCP |
| Server2 | Network K (Switch9) | 72.16.0.2 | 255.255.0.0 | 72.16.0.1 | — |

# 5.  Cabling & Port Mapping

## 5.1 Cable Types Used

| Cable Type | Usage | Color in Packet Tracer |
| --- | --- | --- |
| Serial DCE | Router-to-Router WAN links | Red solid lines |
| Crossover Ethernet | Router10↔Router11, Router11↔Router12 | Black dashed lines |
| Straight-Through | Switch-to-Router, Switch-to-PC/Laptop/Server | Green lines |

## 5.2 WAN Cabling (Router-to-Router)

### OSPF Area 1

| From | Port | To | Port | Cable | WAN |
| --- | --- | --- | --- | --- | --- |
| Router4 | Se0/0/0 | Router1 | Se0/0/0 | Serial DCE | W1 |
| Router1 | Se0/0/1 | Router0 | Se0/0/0 | Serial DCE | W2 |
| Router0 | Se0/0/1 | Router2 | Se0/0/0 | Serial DCE | W3 |
| Router1 | Se0/1/0 | Router2 | Se0/1/0 | Serial DCE | W22 |

### EIGRP 5

| From | Port | To | Port | Cable | WAN |
| --- | --- | --- | --- | --- | --- |
| Router2 | Se0/0/1 | Router3 | Se0/0/0 | Serial DCE | W4 |
| Router3 | Se0/0/1 | Router5 | Se0/0/0 | Serial DCE | W5 |
| Router5 | Se0/0/1 | Router6 | Se0/0/0 | Serial DCE | W6 |
| Router6 | Se0/0/1 | Router7 | Se0/0/0 | Serial DCE | W7 |
| Router6 | Se0/1/0 | Router9 | Se0/0/0 | Serial DCE | W8 |
| Router9 | Se0/0/1 | Router8 | Se0/0/0 | Serial DCE | W9 |
| Router7 | Se0/0/1 | Router8 | Se0/1/0 | Serial DCE | W20 |

### OSPF Area 2

| From | Port | To | Port | Cable | WAN |
| --- | --- | --- | --- | --- | --- |
| Router8 | Se0/0/1 | Router10 | Se0/0/0 | Serial DCE | W10 |
| Router10 | Fa0/0 | Router11 | Fa0/0 | Crossover | W11 |
| Router11 | Fa0/1 | Router12 | Fa0/1 | Crossover | W12 |
| Router11 | Se0/0/0 | Router13 | Se0/0/0 | Serial DCE | W13 |
| Router11 | Se0/0/1 | Router18 | Se0/0/0 | Serial DCE | W14 |

### RIP

| From | Port | To | Port | Cable | WAN |
| --- | --- | --- | --- | --- | --- |
| Router18 | Se0/0/1 | Router14 | Se0/0/0 | Serial DCE | W15 |
| Router14 | Se0/0/1 | Router15 | Se0/0/0 | Serial DCE | W16 |
| Router15 | Se0/0/1 | Router16 | Se0/0/0 | Serial DCE | W17 |
| Router16 | Se0/0/1 | Router17 | Se0/0/0 | Serial DCE | W18 |
| Router15 | Se0/1/0 | Router17 | Se0/0/1 | Serial DCE | W19 |
| Router17 | Se0/1/0 | Router14 | Se0/1/0 | Serial DCE | W21 |

## 5.3 LAN Cabling (Switch-to-Router, all Straight-Through)

| Switch | Switch Port | Router | Router Port |
| --- | --- | --- | --- |
| Switch1 | Fa0/1 | Router4 | Fa0/1 |
| Switch0 | Fa0/1 | Router1 | Fa0/0 |
| Switch2 | Fa0/4 | Router1 | Fa0/1 |
| Switch5 | Fa0/1 | Router7 | Fa0/0 |
| Switch3 | Fa0/1 | Router9 | Fa0/0 |
| Switch4 | Fa0/1 | Router8 | Fa0/0 |
| Switch7 | Fa0/1 | Router12 | Fa0/0 |
| Switch8 | Fa0/1 | Router13 | Fa0/0 |
| Switch6 | Fa0/1 | Router13 | Fa0/1 |
| Switch9 | Fa0/1 | Router16 | Fa0/0 |
| Switch10 | Fa0/1 | Router17 | Fa0/0 |

# 6. Routing Protocol Configurations

## 6.1 OSPF Area 1

OSPF Process ID: 1, Area: 1

### Router4 — Network A Gateway

hostname Router4

interface FastEthernet0/1

ip address 72.2.0.1 255.254.0.0

ip helper-address 72.8.0.2

interface Serial0/0/0

ip address 72.17.0.1 255.255.255.252

router ospf 1

network 72.2.0.0 0.1.255.255 area 1

network 72.17.0.0 0.0.0.3 area 1

### Router1 — Network B+C Gateway (Hub)

interface FastEthernet0/0

ip address 72.12.0.1 255.255.0.0

ip helper-address 72.8.0.2

interface FastEthernet0/1

ip address 72.0.0.1 255.254.0.0

ip helper-address 72.8.0.2

interface Serial0/0/0

ip address 72.17.0.2 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.5 255.255.255.252

interface Serial0/1/0

ip address 72.17.0.85 255.255.255.252

router ospf 1

network 72.12.0.0 0.0.255.255 area 1

network 72.0.0.0 0.1.255.255 area 1

network 72.17.0.0 0.0.0.3 area 1

network 72.17.0.4 0.0.0.3 area 1

network 72.17.0.84 0.0.0.3 area 1

hostname Router0

interface Serial0/0/0

ip address 72.17.0.6 255.255.255.252

### Router0 — Transit

ip address 72.17.0.9 255.255.255.252

router ospf 1

network 72.17.0.4 0.0.0.3 area 1

network 72.17.0.8 0.0.0.3 area 1

hostname Router2

interface Serial0/0/0

ip address 72.17.0.10 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.13 255.255.255.252

### Router2 — OSPF ↔ EIGRP Border (Redistribution)

ip address 72.17.0.86 255.255.255.252

router ospf 1

network 72.17.0.8 0.0.0.3 area 1

network 72.17.0.84 0.0.0.3 area 1

redistribute eigrp 5 subnets

router eigrp 5

network 72.17.0.12 0.0.0.3

redistribute ospf 1 metric 1500 100 255 1 1500

no auto-summary

## 6.2 EIGRP 5

EIGRP Autonomous System: 5

### Router3 — EIGRP Entry

hostname Router3

interface Serial0/0/0

ip address 72.17.0.14 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.17 255.255.255.252

router eigrp 5

network 72.17.0.12 0.0.0.3

network 72.17.0.16 0.0.0.3

no auto-summary

### Router5 — Transit

interface Serial0/0/0

ip address 72.17.0.18 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.21 255.255.255.252

router eigrp 5

network 72.17.0.16 0.0.0.3

network 72.17.0.20 0.0.0.3

no auto-summary

### Router6 — Transit (No LAN)

interface Serial0/0/0

ip address 72.17.0.22 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.25 255.255.255.252

interface Serial0/1/0

ip address 72.17.0.29 255.255.255.252

router eigrp 5

network 72.17.0.20 0.0.0.3

network 72.17.0.24 0.0.0.3

network 72.17.0.28 0.0.0.3

no auto-summary

hostname Router7

### Router7 — Network D Gateway

interface FastEthernet0/0

ip address 72.14.0.1 255.255.0.0

ip helper-address 72.8.0.2

interface Serial0/0/0

ip address 72.17.0.26 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.77 255.255.255.252

router eigrp 5

network 72.14.0.0 0.0.255.255

network 72.17.0.24 0.0.0.3

network 72.17.0.76 0.0.0.3

no auto-summary

hostname Router9

### Router9 — Network E Gateway

interface FastEthernet0/0

ip address 72.6.0.1 255.254.0.0

ip helper-address 72.8.0.2

interface Serial0/0/0

ip address 72.17.0.30 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.33 255.255.255.252

router eigrp 5

network 72.6.0.0 0.1.255.255

network 72.17.0.28 0.0.0.3

network 72.17.0.32 0.0.0.3

no auto-summary

hostname Router8

### Router8 — Network F Gateway + EIGRP→OSPF2 Border

interface FastEthernet0/0

ip address 72.4.0.1 255.254.0.0

ip helper-address 72.8.0.2

interface Serial0/0/0

ip address 72.17.0.34 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.37 255.255.255.252

interface Serial0/1/0

ip address 72.17.0.78 255.255.255.252

router eigrp 5

network 72.4.0.0 0.1.255.255

network 72.17.0.32 0.0.0.3

network 72.17.0.36 0.0.0.3

network 72.17.0.76 0.0.0.3

no auto-summary

## 6.3 OSPF Area 2

### Router10 — EIGRP ↔ OSPF2 Border (Redistribution)

hostname Router10

interface Serial0/0/0

ip address 72.17.0.38 255.255.255.252

interface FastEthernet0/0

ip address 72.17.0.41 255.255.255.252

router eigrp 5

network 72.17.0.36 0.0.0.3

redistribute ospf 1 metric 1500 100 255 1 1500

no auto-summary

router ospf 1

network 72.17.0.40 0.0.0.3 area 2

redistribute eigrp 5 subnets

hostname Router11

### Router11 — OSPF2 Hub

interface FastEthernet0/0

ip address 72.17.0.42 255.255.255.252

interface FastEthernet0/1

ip address 72.17.0.45 255.255.255.252

interface Serial0/0/0

ip address 72.17.0.49 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.53 255.255.255.252

router ospf 1

network 72.17.0.40 0.0.0.3 area 2

network 72.17.0.44 0.0.0.3 area 2

network 72.17.0.48 0.0.0.3 area 2

network 72.17.0.52 0.0.0.3 area 2

hostname Router12

interface FastEthernet0/0

### Router12 — Network G Gateway

ip helper-address 72.8.0.2

interface FastEthernet0/1

ip address 72.17.0.46 255.255.255.252

router ospf 1

network 72.15.0.0 0.0.255.255 area 2

network 72.17.0.44 0.0.0.3 area 2

hostname Router13

interface FastEthernet0/0

### Router13 — Network H+I Gateway

interface FastEthernet0/1

ip address 72.13.0.1 255.255.0.0

ip helper-address 72.8.0.2

interface Serial0/0/0

ip address 72.17.0.50 255.255.255.252

router ospf 1

network 72.8.0.0 0.1.255.255 area 2

network 72.13.0.0 0.0.255.255 area 2

network 72.17.0.48 0.0.0.3 area 2

show ip ospf neighbor on Router11

## 6.4 RIP (Version 2)

### Router18 — OSPF2 ↔ RIP Border (Redistribution)

hostname Router18

interface Serial0/0/0

ip address 72.17.0.54 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.57 255.255.255.252

router ospf 1

network 72.17.0.52 0.0.0.3 area 2

redistribute rip subnets

router rip

version 2

network 72.17.0.56

redistribute ospf 1 metric 5

no auto-summary

### Router14 — Core RIP

interface Serial0/0/0

ip address 72.17.0.58 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.61 255.255.255.252

interface Serial0/1/0

ip address 72.17.0.82 255.255.255.252

router rip

version 2

network 72.17.0.56

network 72.17.0.60

network 72.17.0.80

no auto-summary

hostname Router15

### Router15 — RIP Transit

interface Serial0/0/0

ip address 72.17.0.62 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.65 255.255.255.252

interface Serial0/1/0

ip address 72.17.0.73 255.255.255.252

router rip

version 2

network 72.17.0.60

network 72.17.0.64

network 72.17.0.72

no auto-summary

hostname Router16

### Router16 — Network K Gateway

interface FastEthernet0/0

ip address 72.16.0.1 255.255.0.0

ip helper-address 72.8.0.2

interface Serial0/0/0

ip address 72.17.0.66 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.69 255.255.255.252

router rip

version 2

network 72.16.0.0

network 72.17.0.64

network 72.17.0.68

no auto-summary

hostname Router17

### Router17 — Network J Gateway

ip address 72.10.0.1 255.254.0.0

ip helper-address 72.8.0.2

interface Serial0/0/0

ip address 72.17.0.70 255.255.255.252

interface Serial0/0/1

ip address 72.17.0.74 255.255.255.252

interface Serial0/1/0

ip address 72.17.0.81 255.255.255.252

router rip

version 2

network 72.10.0.0

network 72.17.0.68

network 72.17.0.72

network 72.17.0.80

no auto-summary

# 7. Route Redistribution

Route redistribution allows different routing protocols to share route information across domain boundaries.

| Border Router | From Protocol | To Protocol | Command |
| --- | --- | --- | --- |
| Router2 | EIGRP 5 → OSPF 1 | Under router ospf 1 | redistribute eigrp 5 subnets |
| Router2 | OSPF 1 → EIGRP 5 | Under router eigrp 5 | redistribute ospf 1 metric 1500 100 255 1 1500 |
| Router10 | EIGRP 5 → OSPF 1 | Under router ospf 1 | redistribute eigrp 5 subnets |
| Router10 | OSPF 1 → EIGRP 5 | Under router eigrp 5 | redistribute ospf 1 metric 1500 100 255 1 1500 |
| Router18 | RIP → OSPF 1 | Under router ospf 1 | redistribute rip subnets |
| Router18 | OSPF 1 → RIP | Under router rip | redistribute ospf 1 metric 5 |

EIGRP Metric Format: Bandwidth, Delay, Reliability, Load, MTU (1500 100 255 1 1500)

show ip route on Router10 showing both EIGRP (D) and OSPF (O) routes

# 8. DHCP Configuration

## 8.1 DHCP Server — Server1 (72.8.0.2)

Server1 acts as the centralized DHCP server for all 11 networks. It is located in Network H.

### Server1 Static IP Configuration

| Setting | Value |
| --- | --- |
| IP | 72.8.0.2 |
| Subnet Mask | 255.254.0.0 |
| Gateway | 72.8.0.1 |
| DNS | 72.0.0.2 |

## 8.2 DHCP Pools

| Pool | Network | Default Gateway | DNS Server | Start IP | Subnet Mask |
| --- | --- | --- | --- | --- | --- |
| Net-A | A | 72.2.0.1 | 72.0.0.2 | 72.2.0.10 | 255.254.0.0 |
| Net-B | B | 72.12.0.1 | 72.0.0.2 | 72.12.0.10 | 255.255.0.0 |
| Net-C | C | 72.0.0.1 | 72.0.0.2 | 72.0.0.10 | 255.254.0.0 |
| Net-D | D | 72.14.0.1 | 72.0.0.2 | 72.14.0.10 | 255.255.0.0 |
| Net-E | E | 72.6.0.1 | 72.0.0.2 | 72.6.0.10 | 255.254.0.0 |
| Net-F | F | 72.4.0.1 | 72.0.0.2 | 72.4.0.10 | 255.254.0.0 |
| Net-G | G | 72.15.0.1 | 72.0.0.2 | 72.15.0.10 | 255.255.0.0 |
| Net-I | I | 72.13.0.1 | 72.0.0.2 | 72.13.0.10 | 255.255.0.0 |
| Net-J | J | 72.10.0.1 | 72.0.0.2 | 72.10.0.10 | 255.254.0.0 |
| Net-K | K | 72.16.0.1 | 72.0.0.2 | 72.16.0.10 | 255.255.0.0 |

## 8.3 DHCP Relay (ip helper-address)

Each gateway router forwards DHCP requests to Server1 using ip helper-address 72.8.0.2:

| Router | Interface | Network |
| --- | --- | --- |
| Router4 | Fa0/1 | A |
| Router1 | Fa0/0 | B |
| Router1 | Fa0/1 | C |
| Router7 | Fa0/0 | D |
| Router9 | Fa0/0 | E |
| Router8 | Fa0/0 | F |
| Router12 | Fa0/0 | G |
| Router13 | Fa0/1 | I |
| Router17 | Fa0/0 | J |
| Router16 | Fa0/0 | K |

# 9. DNS, Mail & Web Server Setup

## 9.1 Server0 Configuration — 72.0.0.2 (Network C)

| Setting | Value |
| --- | --- |
| IP Address | 72.0.0.2 |
| Subnet Mask | 255.254.0.0 |
| Default Gateway | 72.0.0.1 |
| DNS Server | 72.0.0.2 (itself) |

## 9.2 DNS Service

| Hostname | Type | Address |
| --- | --- | --- |
| www.project.com | A Record | 72.0.0.2 |

## 9.3 HTTP/Web Service

HTTP Service: ON — Default page serves to any browser accessing http://www.project.com or http://72.0.0.2

## 9.4 Email Service (SMTP/POP3)

Domain: project.com     SMTP: ON     POP3: ON

### Registered Users

| User | Password | Email |
| --- | --- | --- |
| laptop0 | laptop0 | laptop0@project.com |
| laptop1 | laptop1 | laptop1@project.com |
| laptop2 | laptop2 | laptop2@project.com |
| laptop3 | laptop3 | laptop3@project.com |
| pc0 | pc0 | pc0@project.com |
| pc1 | pc1 | pc1@project.com |

## 9.5 Email Client Configuration (Example: Laptop2)

| Field | Value |
| --- | --- |
| Your Name | Laptop2 |
| Email Address | laptop2@project.com |
| Incoming Mail Server | 72.0.0.2 |
| Outgoing Mail Server | 72.0.0.2 |
| User Name | laptop2 |
| Password | laptop2 |

Server0 DNS services

Server0 Email service showing user list

Email send success from Laptop2

Email receive on Laptop0

Web browser on any PC accessing www.project.com

# 10. NAT Configuration

NAT is configured on Router17 (gateway for Network J) to translate the private IP address to the public IP address.

## 10.1 NAT Details

| Setting | Value |
| --- | --- |
| Inside Local | 1.139.135.189 |
| Inside Global (Public) | 72.195.221.244 |
| NAT Type | Static |
| Inside Interface | Fa0/0 (Network J) |
| Outside Interfaces | Se0/0/0, Se0/0/1, Se0/1/0 (WAN serial links) |

## 10.2 NAT Commands on Router17

interface FastEthernet0/0

ip nat inside

interface Serial0/0/0

ip nat outside

interface Serial0/0/1

ip nat outside

interface Serial0/1/0

ip nat outside

ip nat inside source static 1.139.135.189 72.195.221.244

## 10.3 Verification

Router17#show ip nat translations

Pro  Inside global      Inside local        Outside local    Outside global

---  72.195.221.244     1.139.135.189       ---              ---

Router17#show ip nat statistics

Total translations: 1 (1 static, 0 dynamic)

Outside Interfaces: Serial0/0/0, Serial0/0/1, Serial0/1/0

Inside Interfaces: FastEthernet0/0

show ip nat translations output on Router17

show ip nat statistics output on Router17

# 11. Access Control Lists (ACLs)

## 11.1 ACL 1: Block ONE Host in Network A from Web Server

Purpose: Prevent a specific host (first DHCP-assigned PC) in Network A from accessing Server0.

| Setting | Value |
| --- | --- |
| Applied On | Router4 |
| Interface | Fa0/1 (inbound) |
| Blocked Source | 72.2.0.10 (first DHCP host) |
| Blocked Destination | 72.0.0.2 (Server0) |

access-list 100 deny   ip host 72.2.0.10 host 72.0.0.2

access-list 100 permit ip any any

interface FastEthernet0/1

ip access-group 100 in

## 11.2 ACL 2: Block ALL of Network D from Web Server

Purpose: Prevent the entire Network D subnet from accessing Server0.

| Setting | Value |
| --- | --- |
| Applied On | Router7 |
| Interface | Fa0/0 (inbound) |
| Blocked Source | 72.14.0.0/16 (entire Network D) |
| Blocked Destination | 72.0.0.2 (Server0) |

access-list 101 deny   ip 72.14.0.0 0.0.255.255 host 72.0.0.2

access-list 101 permit ip any any

interface FastEthernet0/0

ip access-group 101 in

show access-lists on Router4

show access-lists on Router7

Ping from blocked host (72.2.0.11) to 72.0.0.2 — FAILING]

Ping from non-blocked host (72.2.0.10) in Network A to 72.0.0.2 — SUCCESS]

Ping from Network D host to 72.0.0.2 — FAILING

# 12. Testing & Verification

## 12.1 Routing Table Verification

Run show ip route on key routers to verify all networks are reachable:

| Router | Expected Route Types |
| --- | --- |
| Router2 | O (OSPF), D EX (EIGRP external) |
| Router10 | D (EIGRP), O (OSPF), O E2 (OSPF external) |
| Router18 | O (OSPF), R (RIP) |
| Router17 | R (RIP routes to all networks) |

show ip route on Router2

show ip route on Router18

## 12.2 End-to-End Ping Tests

| From | To | Expected | Why |
| --- | --- | --- | --- |
| Laptop3 (Net A) | 72.0.0.2 (Server0) | Success | Normal connectivity |
| Laptop0 (Net B) | 72.10.0.1 (Net J gateway) | Success | Cross-domain via redistribution |
| PC4 (Net D) | 72.0.0.2 (Server0) | Fail | ACL blocks Network D |
| PC7 (Net G) | 72.2.0.1 (Net A gateway) | Success | OSPF2 to EIGRP to OSPF1 |
| PC12 (Net J) | 72.15.0.1 (Net G gateway) | Success | RIP to OSPF2 |

Ping from Network D to Server0 — blocked by ACL

## 12.3 DHCP Verification

| Device | Network | Expected IP Range | Gateway |
| --- | --- | --- | --- |
| Laptop2 | A | 72.2.0.10+ | 72.2.0.1 |
| Laptop0 | B | 72.12.0.10+ | 72.12.0.1 |
| PC0 | C | 72.0.0.10+ | 72.0.0.1 |
| PC4 | D | 72.14.0.10+ | 72.14.0.1 |
| PC7 | G | 72.15.0.10+ | 72.15.0.1 |
| PC12 | J | 72.10.0.10+ | 72.10.0.1 |

IP Configuration of Laptop 3 showing DHCP-assigned IP

IP Configuration of PC 0 showing DHCP-assigned IP

IP Configuration of PC 2 showing DHCP-assigned IP

## 12.4 Service Tests

| Test | How | Expected |
| --- | --- | --- |
| DNS | PC → Web Browser → www.project.com | Website loads |
| Web | PC → Web Browser → http://72.0.0.2 | Website loads |
| Email Send | Laptop3 → Email → Compose → laptop0@project.com | Send success |
| Email Receive | Laptop0 → Email → Receive | Email appears |

Web browser showing www.project.com

Email compose and send success

Email received on Laptop0

# 13. Conclusion

This project successfully demonstrates the design and implementation of a multi-area enterprise network using Cisco Packet Tracer. Key achievements include:

- VLSM Subnetting: Efficiently allocated 11 networks and 22 WAN links from a single /8 address space, minimizing IP waste.

- Multi-Protocol Routing: Successfully configured OSPF (Areas 1 & 2), EIGRP (AS 5), and RIPv2, each serving distinct network segments.

- Route Redistribution: Implemented mutual redistribution at three border routers (Router2, Router10, Router18), enabling full end-to-end connectivity across all routing domains.

- Centralized DHCP: Configured Server1 as a centralized DHCP server with relay agents on all gateway routers, providing automatic IP assignment to all end devices.

- Network Services: Deployed DNS, HTTP, and SMTP/POP3 email services on Server0, enabling web browsing and email communication across the network.

- Security: Applied static NAT on Router17 (Network J) for address translation, and extended ACLs to restrict specific hosts and entire subnets from accessing the web server.

- Redundancy: Multiple parallel paths exist (Router1 to Router2 direct link, Router7 to Router8, Router15 to Router17, Router17 to Router14) providing network resilience.

The network passed all verification tests including cross-domain pings, DHCP assignment, email send/receive, web access, NAT translation, and ACL enforcement.

Prepared by: Alishba Inam (24I-2036)

Date: 6 May 2026